FPGA-Based Sobel Filter with 2×2 Average Pooling

Block Diagram :

FPGA

PC

Gray scale image (.hex)

Output to image conversion

BRAM 
(64X64)

Input controller

FIFO

3X3 Window

Line Buffers

Sobel Filter

2X2 AVG pooling layer

1. Prerequisites

- A grayscale image is converted into hexadecimal (.hex) format before processing.

- The image data is preloaded into BRAM, configured to operate at 200 MHz.

- CDC : The design uses an asynchronous FIFO to transfer data between the 100 MHz write clock domain and the 200 MHz read clock domain.

2. BRAM and FIFO Operation

- BRAM: Stores the preloaded grayscale image data and supplies it to the input controller.

- Asynchronous FIFO: Has a depth of 32 locations and is used for clock-domain crossing.

- Write clock (WR_CLK): 100 MHz.

- Read clock (RD_CLK): 200 MHz.

- The input controller manages data transfer from BRAM to the FIFO, and the FIFO provides data to the image-processing pipeline in the read-clock domain.

3. Line Buffers and 3×3 Window Generation

The data read from the FIFO is fed into the line buffers and a 3×3 shift register. The line buffers retain the required pixel data from consecutive image rows, while the shift register maintains a sliding 3×3 pixel window. This window provides the required input matrix for the Sobel filter.

4. Sobel Filter Operation

The Sobel filter processes the 3×3 pixel window to calculate the horizontal gradient (\(G_x\)) and vertical gradient (\(G_y\)). These gradients are combined to produce the output gradient value \(G\), according to the implemented RTL logic.

Whenever a valid Sobel output is available, the G_rdy signal is asserted for 1 cycle to indicate that the output can be consumed by the pooling layer.

5. 2×2 Average Pooling

The pooling layer uses its own line buffers to collect valid Sobel outputs and form a 2×2 window. Once four valid values are available, the layer calculates their average:

\[ PG=\frac{G_{11}+G_{12}+G_{21}+G_{22}}{4} \]

The PG_rdy signal is asserted for 1 cycle whenever a valid pooled output is available. The resulting pooled data is forwarded to the output stage for image conversion.

6. Overall Data Flow

The complete processing sequence is:

Grayscale .hex Image → BRAM (200 MHz) → Input Controller → Asynchronous FIFO (32 locations) → Line Buffers → 3×3 Window → Sobel Filter → 2×2 Average Pooling → Image Conversion

The design uses clock-domain crossing, line buffering, sliding-window generation, and valid-data handshaking to implement the Sobel edge-detection and average-pooling pipeline on the FPGA.